{

"nbformat": 4,

"nbformat_minor": 0,

"metadata": {

"colab": {

"provenance": []

},

"kernelspec": {

"name": "python3",

"display_name": "Python 3"

},

"language_info": {

"name": "python"

}

},

"cells": [

{

"cell_type": "markdown",

"source": [

"**EXP NO 6 -LINEAR BLOCK CODE**\n",

"\n",

"**NAME : AYESHA M**\n",

"\n",

"**REG NO : 212224060034**"

],

"metadata": {

"id": "6BMXoMZ40QIS"

}

},

{

"cell_type": "markdown",

"source": [],

"metadata": {

"id": "4-RMB5xM0jbf"

}

},

{

"cell_type": "code",

"source": [

"import numpy as np\n",

"\n",

"pb = []\n",

"col = int(input(\"Enter the Parity bits : \"))      # n-k\n",

"row = int(input(\"Enter the Message bits : \"))     # k\n",

"\n",

"# -----------------------------\n",

"# Construct Parity Matrix P\n",

"# -----------------------------\n",

"for i in range(row):\n",

"    p = list(map(int, input(f\"Enter row {i+1} (space separated) : \").split()))\n",

"    pb.append(p)\n",

"\n",

"P = np.array(pb, dtype=int)\n",

"\n",

"# -----------------------------\n",

"# Generator Matrix G = [P | I]\n",

"# -----------------------------\n",

"I_k = np.eye(row, dtype=int)\n",

"G = np.hstack((P, I_k))\n",

"\n",

"print(\"\\nGenerator Matrix (G):\")\n",

"for r in G:\n",

"    print(\" \".join(map(str, r)))\n",

"\n",

"# Code parameters\n",

"k = row\n",

"n = G.shape[1]\n",

"\n",

"# -----------------------------\n",

"# Generate all possible messages\n",

"# -----------------------------\n",

"messages = np.array([[int(x) for x in format(i, f'0{k}b')] for i in range(2**k)])\n",

"\n",

"# Generate codewords\n",

"codewords = np.mod(np.dot(messages, G), 2)\n",

"\n",

"print(\"\\nMessage   Codeword   Weight\")\n",

"weights = []\n",

"\n",

"for i in range(len(messages)):\n",

"    wt = np.sum(codewords[i])\n",

"    weights.append(wt)\n",

"    print(\" \".join(map(str, messages[i])), \"   \",\n",

"          \" \".join(map(str, codewords[i])), \"   \", wt)\n",

"\n",

"# Minimum Hamming distance\n",

"d_min = np.min([w for w in weights if w != 0])\n",

"print(\"\\nMinimum Hamming Distance:\", d_min)\n",

"\n",

"# -----------------------------\n",

"# Parity Check Matrix H = [I | P^T]\n",

"# -----------------------------\n",

"I_r = np.eye(col, dtype=int)\n",

"H = np.hstack((I_r, P.T))\n",

"\n",

"print(\"\\nParity Check Matrix (H):\")\n",

"for r in H:\n",

"    print(\" \".join(map(str, r)))\n",

"\n",

"H_T = H.T\n",

"\n",

"# -----------------------------\n",

"# Receive Codeword\n",

"# -----------------------------\n",

"rc = list(map(int, input(\"\\nEnter the received codeword : \").split()))\n",

"r = np.array(rc)\n",

"\n",

"# Syndrome\n",

"S = np.mod(np.dot(r, H_T), 2)\n",

"\n",

"print(\"Syndrome :\", \" \".join(map(str, S)))\n",

"\n",

"# -----------------------------\n",

"# Error Detection & Correction\n",

"# -----------------------------\n",

"error = np.zeros(n, dtype=int)\n",

"\n",

"for i in range(n):\n",

"    if np.array_equal(H_T[i], S):\n",

"        error[i] = 1\n",

"        break\n",

"\n",

"print(\"Error Vector :\", \" \".join(map(str, error)))\n",

"\n",

"# Corrected codeword\n",

"corrected = np.mod(r + error, 2)\n",

"print(\"Corrected Codeword :\", \" \".join(map(str, corrected)))"

],

"metadata": {

"colab": {

"base_uri": "https://localhost:8080/"

},

"id": "luqjegONz96F",

"outputId": "ac16d225-0084-4f68-c9e8-6c5e770151d3"

},

"execution_count": 4,

"outputs": [

{

"output_type": "stream",

"name": "stdout",

"text": [

"Enter the Parity bits : 3\n",

"Enter the Message bits : 3\n",

"Enter row 1 (space separated) : 1 1 0\n",

"Enter row 2 (space separated) : 0 1 1\n",

"Enter row 3 (space separated) : 1 0 1\n",

"\n",

"Generator Matrix (G):\n",

"1 1 0 1 0 0\n",

"0 1 1 0 1 0\n",

"1 0 1 0 0 1\n",

"\n",

"Message   Codeword   Weight\n",

"0 0 0     0 0 0 0 0 0     0\n",

"0 0 1     1 0 1 0 0 1     3\n",

"0 1 0     0 1 1 0 1 0     3\n",

"0 1 1     1 1 0 0 1 1     4\n",

"1 0 0     1 1 0 1 0 0     3\n",

"1 0 1     0 1 1 1 0 1     4\n",

"1 1 0     1 0 1 1 1 0     4\n",

"1 1 1     0 0 0 1 1 1     3\n",

"\n",

"Minimum Hamming Distance: 3\n",

"\n",

"Parity Check Matrix (H):\n",

"1 0 0 1 0 1\n",

"0 1 0 1 1 0\n",

"0 0 1 0 1 1\n",

"\n",

"Enter the received codeword : 1 0 0 1 1 1\n",

"Syndrome : 1 0 0\n",

"Error Vector : 1 0 0 0 0 0\n",

"Corrected Codeword : 0 0 0 1 1 1\n"

]

}

]

}

]

}